// SMART DOOR LOCK — Phase 1

#include <Servo.h>

const int BUTTON_PIN = 2;

const int GREEN_LED  = 3;

const int RED_LED    = 4;

const int SERVO_PIN  = 9;

Servo doorLock;

bool isLocked = true;

bool lastButtonState = LOW;

// How long door stays unlocked before auto-locking (milliseconds)

const int AUTO_LOCK_DELAY = 3000; // 3 seconds

void setup() {

pinMode(BUTTON_PIN, INPUT);

pinMode(GREEN_LED, OUTPUT);

pinMode(RED_LED, OUTPUT);

doorLock.attach(SERVO_PIN);

Serial.begin(9600);

updateLockState();

}

void loop() {

bool currentButton = digitalRead(BUTTON_PIN);

// Detect clean single press (LOW → HIGH transition only)

if (currentButton == HIGH && lastButtonState == LOW) {

if (isLocked) {

// === UNLOCK ===

isLocked = false;

updateLockState();

Serial.println("Door unlocked! Auto-locking in 3 seconds...");

// Wait 3 seconds with door open, then auto-lock

delay(AUTO_LOCK_DELAY);

// === AUTO LOCK ===

isLocked = true;

updateLockState();

Serial.println("Auto-locked!");

} else {

// If already unlocked and button pressed again → lock immediately

isLocked = true;

updateLockState();

}

// Long debounce — waits for finger to lift off button

delay(500);

}

lastButtonState = currentButton;

}

void updateLockState() {

if (isLocked) {

digitalWrite(RED_LED, HIGH);

digitalWrite(GREEN_LED, LOW);

doorLock.write(0);        // 0° = LOCKED

Serial.println(">> LOCKED");

} else {

digitalWrite(RED_LED, LOW);

digitalWrite(GREEN_LED, HIGH);

doorLock.write(90);       // 90° = UNLOCKED

Serial.println(">> UNLOCKED");

}

}