SMART SECURITY DOOR LOCK SYSTEM

Project Documentation — 3-Phase Arduino Development

Phase 1: Tinkercad Prototype   |   Phase 2: Wokwi Simulation   |   Phase 3: Real Implementation

| Project Title | Smart Security Door Lock System |
| --- | --- |
| Development Stages | 3 Phases (Prototype → Simulation → Real Build) |
| Microcontrollers | Arduino UNO (Phase 1 & 3), Arduino Mega (Phase 2) |
| Simulation Tools | Tinkercad (Phase 1), Wokwi (Phase 2) |
| Key Technologies | RFID RC522, SPI, I2C LCD, Keypad, Servo, EEPROM |
| Languages / Libraries | C++ (Arduino IDE), SPI, MFRC522, LiquidCrystal_I2C, Keypad |

# Table of Contents

1.  Project Overview	3

2.  Phase 1 — Basic Prototype (Tinkercad)	4

2.1  Components & Wiring	4

2.2  How It Works	5

2.3  Code Highlights	5

3.  Phase 2 — Full Simulation (Wokwi / Arduino Mega)	6

3.1  Components & Wiring	6

3.2  System Features	7

3.3  State Machine	8

3.4  Code Highlights	9

4.  Phase 3 — Real-Life Implementation (Arduino UNO)	10

4.1  Hardware Components	10

4.2  UNO Adaptations	11

4.3  Real Build Photo	12

4.4  Code Highlights	12

5.  Phase Comparison	13

6.  Conclusions & Future Work	14

# 1. Project Overview

This project documents a progressive three-phase development of an Arduino-based Smart Security Door Lock. Each phase builds upon the previous, adding features, authentication methods, and real-world robustness.

| Phase | Platform | Controller | Auth Method | Key Features |
| --- | --- | --- | --- | --- |
| Phase 1 | Tinkercad | Arduino UNO | Push Button | Single-user, LED, Servo, Auto-lock |
| Phase 2 | Wokwi | Arduino Mega | RFID + PIN | Multi-user, LCD, EEPROM, Admin Menu |
| Phase 3 | Real Hardware | Arduino UNO | RFID + PIN | Full features on physical build |

# 2. Phase 1 — Basic Prototype (Tinkercad)

The first phase established the core concept: a push-button-controlled door lock simulated entirely in Tinkercad with an Arduino UNO. It proves the servo-as-bolt mechanism and LED feedback loop in a zero-risk environment.

## 2.1  Components & Wiring

| Arduino UNO | Main microcontroller |
| --- | --- |
| Push Button | Digital Pin 2 — lock/unlock trigger |
| Red LED | Digital Pin 4 — locked indicator |
| Green LED | Digital Pin 3 — unlocked indicator |
| Servo Motor | Digital Pin 9 — 0° = Locked, 90° = Unlocked |
| Resistors | 220 Ω for each LED; 10 kΩ pull-down for button |

## 2.2  How It Works

- System starts in LOCKED state: Red LED ON, servo at 0°.

- A LOW → HIGH transition on Pin 2 triggers the unlock sequence.

- Servo rotates to 90°, Red LED turns OFF, Green LED turns ON.

- After 3 000 ms the system automatically re-locks (servo back to 0°).

- A second button press while unlocked immediately re-locks the door.

- All state changes are logged to the Serial Monitor at 9600 baud.

## 2.3  Code Highlights

// Key constants

const int BUTTON_PIN = 2;  const int GREEN_LED = 3;

const int RED_LED    = 4;  const int SERVO_PIN  = 9;

const int AUTO_LOCK_DELAY = 3000; // 3 seconds

// Unlock → auto-lock flow

if (currentButton == HIGH && lastButtonState == LOW) {

isLocked = false;  updateLockState();

delay(AUTO_LOCK_DELAY);   // hold door open

isLocked = true;   updateLockState(); // auto-lock

}

// updateLockState() drives LEDs + servo

doorLock.write(isLocked ? 0 : 90); // 0°=LOCKED  90°=UNLOCKED

# 3. Phase 2 — Full Simulation (Wokwi / Arduino Mega)

Phase 2 elevated the project to a fully-featured security system simulated in Wokwi. The Arduino Mega provided ample I/O for RFID, a 4×4 keypad, LCD, and buzzer — all integrated with EEPROM persistence and multi-user role management.

## 3.1  Components & Wiring

| Arduino Mega 2560 | Main microcontroller (SPI + I2C + ample digital I/O) |
| --- | --- |
| MFRC522 RFID | SS=53, RST=49 via SPI bus |
| 4×4 Keypad | Rows: pins 22–25 Cols: pins 26–29 |
| LCD 16×2 (I2C) | I2C address 0x27 |
| Servo Motor | Pin 6 — LOCK_POS=0° UNLOCK_POS=90° |
| Buzzer | Pin 7 — differentiated tones per user role |
| Red LED | Pin 5 — access denied / locked |
| Green LED | Pin 4 — access granted / unlocked |

## 3.2  System Features

RFID Authentication:  Scan any of 4 registered cards; UID match triggers access.

PIN Entry:  4–8 digit PIN via keypad; # confirms, * clears buffer.

4 User Roles:  Admin (Youssef) · User Ahmed · User Karim · Guest — each with unique PIN & card UID.

Lockout System:  6 consecutive failed attempts triggers a 30-second lockout with alarm sound and flashing Red LED.

Admin Override:  Pressing * during lockout enters Admin Login mode; correct Admin PIN unlocks the system.

EEPROM Persistence:  PINs, UIDs, lockout state, and silent-mode preference saved across power cycles.

Admin Menu:  1 = Reset lockout  |  2 = Edit users  |  3 = Toggle sound  |  * = Exit

Panic Code (911):  Entering '911' as a PIN fires an emergency alarm sequence without unlocking the door.

Silent Mode:  Admin can mute all buzzer feedback via the menu.

Auto-lock:  Door re-locks automatically after DOOR_OPEN_TIME (3 000 ms).

## 3.3  State Machine

The system operates across seven distinct states, each handling a specific mode of user interaction:

| State | Description | Entry Condition |
| --- | --- | --- |
| SYS_IDLE | Awaiting RFID scan or PIN input | Default / access granted |
| SYS_LOCKED | Lockout active — input blocked | 6 failed attempts |
| SYS_ADMIN_LOGIN | Admin PIN entry | * pressed (idle or lockout) |
| SYS_ADMIN_MENU | Admin control menu (1/2/3/*) | Correct Admin PIN |
| SYS_ROLE_SELECT | Choose Admin/User/Guest to edit | Admin menu → 2 |
| SYS_USER_EDIT | Edit PIN or toggle user active | Role selected |
| SYS_CHANGE_PIN | New PIN input for selected user | User edit → 1 |

## 3.4  Code Highlights

// User struct — role-based access control

struct User { byte uid[4]; const char* name; char pin[9]; Role role; bool active; };

// Grant access

void grantAccess(byte userIndex) {

openDoor();  flashGreen(2);  loginSound(userIndex);

delay(DOOR_OPEN_TIME);  closeDoor();

failedAttempts = 0;  saveSystemEEPROM();

}

// Trigger lockout after MAX_FAILED_ATTEMPTS

void triggerLockout() {

lockedOut = true;  lockoutEnd = millis() + LOCKOUT_TIME;

currentState = SYS_LOCKED;  alarmSound();

}

// EEPROM persistence

EEPROM.update(EEPROM_ATTEMPTS, failedAttempts);

EEPROM.update(EEPROM_LOCKED,   lockedOut ? 1 : 0);

# 4. Phase 3 — Real-Life Implementation (Arduino UNO)

Phase 3 brings the simulation to life on physical hardware. The Arduino UNO's limited resources required several adaptations, while new features were added: smooth servo motion, LCD custom characters, RFID card assignment via menu, and an RFID card override during lockout.

## 4.1  Hardware Components

| Arduino UNO | Main microcontroller (2 KB SRAM, 32 KB Flash) |
| --- | --- |
| RFID RC522 | SS=10, RST=9 via SPI bus |
| 3×4 Keypad | Rows: 2,3,8,A0 Cols: A1,A2,A3 |
| LCD 16×2 (I2C) | Address 0x27 — lock/unlock custom icons |
| Servo Motor | Pin 6 — LOCK_POS=90° UNLOCK_POS=0° (reversed from Phase 2) |
| Buzzer | Pin 7 — differentiated melodies per user role |
| Red LED | Pin 5 — access denied / locked state |
| Green LED | Pin 4 — access granted / door open |
| Breadboard | Houses LEDs, resistors, wiring |

## 4.2  UNO Adaptations vs Mega

| Feature | Phase 2 (Mega) | Phase 3 (UNO) | Reason / Notes |
| --- | --- | --- | --- |
| Keypad layout | 4×4 (A–D row) | 3×4 (no letter keys) | UNO pin count |
| Pin routing | Ports 22–29 | Analog A0–A3 as digital | Only 14 digital I/O |
| Servo motion | Instant write() | Smooth ramp ±2° | Realistic door feel |
| LCD icons | None | Custom lock/unlock chars | lcd.createChar() |
| Card assignment | UID hardcoded | Admin menu → Set Card | Dynamic registration |
| User selection | Direct key | Role → User 2-step | Extra SYS_USER_SELECT state |
| Memory | 8 KB SRAM | F() macro for strings | Prevents SRAM overflow |
| Lockout override | PIN only | PIN + RFID card | Faster admin recovery |

## 4.3  Real Build Photo

LCD showing 'SCAN CARD / OR ENTER PIN' — the system idle state.

## 4.4  Code Highlights

// Smooth servo ramp — unique to Phase 3

void openDoor() {

doorServo.attach(SERVO_PIN);

for (int p = LOCK_POS; p >= UNLOCK_POS; p -= 2) {

doorServo.write(p);  delay(10);

}

delay(200);  doorServo.detach();

}

// F() macro saves 2 KB SRAM

show(F("SCAN CARD"), F("OR ENTER PIN"));

// RFID admin card override during lockout

if (uidBelongsToAdmin(mfrc522.uid.uidByte, mfrc522.uid.size)) {

clearLockout();

show(F("RFID OVERRIDE"), F("LOCK CLEARED"));

}

// Custom LCD lock icon

byte lockIcon[8] = {B01110,B10001,B10001,B11111,B11011,B11011,B11111,B00000};

lcd.createChar(0, lockIcon);

# 5. Phase Comparison

| Feature | Phase 1 | Phase 2 | Phase 3 |
| --- | --- | --- | --- |
| Microcontroller | Arduino UNO | Arduino Mega | Arduino UNO |
| Platform | Tinkercad | Wokwi Simulator | Real Hardware |
| Auth Method | Push Button | RFID + PIN | RFID + PIN |
| Display | Serial Monitor | LCD 16×2 I2C | LCD 16×2 I2C |
| Keypad | None | 4×4 Matrix | 3×4 Matrix |
| Users | 1 (implicit) | 4 (Admin/User/Guest) | 4 (Admin/User/Guest) |
| EEPROM Persist | No | Yes | Yes |
| Lockout | No | Yes (30 s) | Yes (30 s) |
| Admin Menu | No | Yes | Yes |
| Panic Code | No | Yes (911) | Yes (911) |
| Card Assignment | No | No | Yes (via menu) |
| RFID Lockout Override | No | No (PIN only) | Yes (PIN or card) |
| Smooth Servo | No | No | Yes (2° ramp) |
| Custom LCD Icons | No | No | Yes (lock/unlock) |
| Flash Strings (F()) | No | No | Yes (SRAM saving) |

# 6. Conclusions & Future Work

## 6.1  What Was Achieved

- Phase 1 validated the core servo-and-LED lock mechanism with minimal components.

- Phase 2 proved the full security architecture — RFID, PIN, roles, lockout, admin menu — in a zero-risk simulated environment on the more capable Mega.

- Phase 3 successfully migrated the full feature set to real hardware on a resource-constrained UNO, requiring SRAM optimisation, pin remapping, and creative I/O multiplexing via analog pins.

- EEPROM persistence ensures PINs, UIDs, and system state survive power cycles.

- Role-based access, layered lockout, admin override, and panic code provide defence-in-depth.

## 6.2  Key Lessons Learned

- Simulation-first development (Tinkercad → Wokwi) dramatically reduced debugging time on real hardware.

- The UNO's 2 KB SRAM requires disciplined use of the F() macro for string literals.

- Smooth servo motion significantly improves perceived build quality with no functional trade-off.

- RFID lockout override (Phase 3) makes the system more practical in a real security setting.

## 6.3  Future Work

- WiFi/BLE integration: remote unlock via smartphone app (e.g., ESP32 upgrade).

- OTA PIN management: change PINs without physical keypad access.

- Access logging: store timestamped access events to SD card or cloud.

- Camera module: capture photo on failed access attempts.

- Biometric auth: fingerprint sensor as third authentication factor.

- Enclosure design: 3D-printed housing to protect electronics and servo mechanism.

Smart Security Door Lock System  —  End of Documentation